Army Offline Firmware – USB Export Setup Guide

Purpose

This guide explains how to enable USB export functionality for the updated Army Offline Firmware.

Step 1: Prepare the USB Drive

Insert a USB drive or an SD card (using a USB card reader) into your Windows PC.

Open PowerShell and create an empty file named BLINKSMART_EXPORT in the root of the USB drive.

Option 1

New-Item -Path E:\BLINKSMART_EXPORT -ItemType File

Option 2

"" | Out-File -FilePath E:\BLINKSMART_EXPORT -NoNewline

If the file already exists, overwrite it using:

New-Item -Path E:\BLINKSMART_EXPORT -ItemType File -Force

Note: Replace E: with your USB drive letter if it is different.

Step 2: Verify the USB

Run:

Get-ChildItem E:\

Expected output:

Directory: E:\

Mode                 LastWriteTime         Length Name

----                 -------------         ------ ----

-a----        ...                         0      BLINKSMART_EXPORT

Verify that:

- The file name is exactly BLINKSMART_EXPORT

- The file has no extension

- The file size is 0 bytes

- The file is located in the root of the USB drive

Step 3: Connect to the Raspberry Pi

SSH into the Raspberry Pi.

Navigate to the firmware directory:

cd /home/pi/facial-tracker-firmware

(Or navigate to your deployed firmware directory if it is different.)

Step 4: Install USB Export Service

Run:

sudo bash setup/install-usb-export.sh

This installs:

- USB detection (udev) rules

- USB export systemd service

- Automatic export functionality when a prepared USB drive is inserted

Step 5: Verify Installation

Check if the udev rule is installed

ls /etc/udev/rules.d/99-blinksmart-usb-export.rules

Check if the systemd service is installed

systemctl cat 'blinksmart-usb-export@.service'

Step 6: Monitor Automatic USB Export

To watch the service in real time whenever a prepared USB drive is connected, run:

journalctl -u 'blinksmart-usb-export@*' -f

Insert the prepared USB drive.

If everything is configured correctly, the service will automatically detect the USB containing the BLINKSMART_EXPORT marker file and start the export process.

Summary

- Prepare the USB by creating an empty file named BLINKSMART_EXPORT in the root directory.

- Verify the file exists and is 0 bytes.

- SSH into the Raspberry Pi.

- Navigate to the firmware directory.

- Run sudo bash setup/install-usb-export.sh.

- Verify the installation.

- Monitor the logs using journalctl.

- Insert the prepared USB drive to trigger automatic export.